Basic steps to get from raw data to raster maps

- Dave give me files containing header information and data.

- Cross-reference files to see which records have associated data.  Only input records with data into the compiled spreadsheets and access database.

- Get lat/longs for records via SQL Plus or ArcMap using the GISDB_IL_Wells_Boring_Location_Pt file from oradbs.  Make sure lat/longs are stored as numbers.

- Put the data and header information into the proper quad folder’s outcrop excel file (ex: Sinkhole>quaternary>Ostendorf>Quadrangle_Strat_Project>Elsah>ELS_outcrops.xls).  Make sure the API numbers are stored as text and numerical data is stored as numbers.

- Copy and paste header and data into the proper compiled spreadsheets.

  - Note that if the clay mineralogy data is stored as GE, it must be converted to Scintag.    Copy and paste the equation for Expandables, Illite, and K+C.  Make sure the equation is referencing the proper cells.

- In the access database, go to database tools and click the relationships button.  Remove all relationships prior to updating the database.

- Copy and paste compiled spreadsheets into the proper access database tables.  This needs to be done one column at a time (you can do several at a time, but I’ve found it leads to more mistakes).  Make sure all records properly copy.

- Go back to database tools>relationships.   Click Header_Location API and drag and let go on top of another table’s API.  The “Edit Relationships” box will popup.  Make sure that both columns say API, click enforce referential integrity.

  - Note: If you get an error, it will likely be due to one of the tables missing or having a typo in the API.  You will need to comb through to find the problem.

- Update the queries.

  - Double click a query.  Go to the design view.  The queries should all be set up.  Hit Run. The queries vary a bit based on what we are trying to find but each query should have columns that look like this:

| Field | Unit | Texture | Depth |
| --- | --- | --- | --- |
| Table | Short_Description | Short_Description | Particle_Size |
| Sort |  |  |  |
| Show | “checked” | “checked” | “checked” |
| Criteria | Like “*banner*” | Like “*D*” | >=[Short_Description].[Top] And <=[Short_Description].[Bottom] |
| Or | Like “*omph*” | Like “*D*” | >=[Short_Description].[Top] And <=[Short_Description].[Bottom] |

- When you are updating the queries you will need to run the queries twice.  Once, as above, with Like “*banner*” and Like “*omph*” and a second time with Like “*Glasford*”.

  - Note that you may need to delete a row if the query is saved with Like “*banner*” and Like “*omph*.  If the query is saved as Like “*Glasford*” then you will need to copy all of the criteria information for each column into the 2nd row.

- Copy query results and paste into proper excel file (ex: Sinkhole>quaternary>Ostendorf>Quadrangle_Strat_Project>ALL_QUADS_LAB_DATA>Query Results>Unit_Diamict_Carbonate.xls)

- Delete the results that contain any of the following: B horizon, inclusion, sand lens, weathered (wx), Harkness Silt.

- Update stats in the excel file.

- Update comparison of Glasford and banner file.  Open Comparison of Glasford & Banner excel file (Sinkhole>quaternary>Ostendorf>Quadrangle_Strat_Project>ALL_QUADS_LAB_DATA>Query Results>Comparison of Glasford & Banner>COMPARISON_OF_GLASFORD_BANNER.xlsx).  Click enable content.  Update all tabs & graphs to reflect the new query results.

- Make a pdf of the updated comparison file.  Go to File>Print.  Under Printer select Adobe PDF.  Under settings select Print Entire Workbook.  It will likely have to divide the workbook up into parts.  Save each part into a dated folder in the Comparison of Glasford & Banner file.

- Make shapefiles out of query data.

  - Go to Tools>Add XY data.  Click the folder and navigate to one of your query result files, select either the Glasford of banner file.  Click OK.

  - Specify the fields for the X and Y coordinates (DD_N83_X and DD_N83_Y).

  - Define your coordinate system.  Click Edit…>Select…>Geographic Coordinate Systems>North America>North American Datum 1983.prj>Add>OK.

  - Make XY file into shapefile.  Right click file>Data>Export Data...either overwrite the existing shapefile or make a new file.

  - Put shapefile in proper group and subgroup.

- Make raster map from the shapefile.

  - Open the attribute table of the shapefile.  Check to see that there are no empty rows at the bottom of the table.  If so, delete them, or better yet, edit your excel query table so that when you import it into ArcMap it won’t import blank rows.  Also, check that each row has an associated x-y coordinate.  Zoom to layer, if the file extends beyond Illinois, then there’s a problem with the x-y data.

  - Open Arc Toolbox.  Spatial Analyst>Interpolation>Topo to Raster.

    - Select a shapefile.

    - In the Field column select the data from which you want a map.

    - In the Type column select Point Elevation.

    - In the output surface raster file, you may either save it as a new file or overwrite an existing file.

    - Drainage enforcement: No Enforce

    - Primary type of input data: Spot

    - Discretization error factor: 1

    - Vertical standard error: 1

    - Tolerance 1: 1

    - Click OK

  - Note that by following the procedures above the Dolomite and Calcite maps will display false zero values for records that don’t have any data.  To make a map that removes these false zeros you must perform a definition query.

    - Right click Glasford/Banner_Diamict_Carbonate_Calcareous shapefile, go to definition query.

    - Type “Method” = ‘Chittick’

    - Remember to remove this definition query once the dolomite and calcite maps are made.

Procedure notes by Lisa Colville, 2012